# MOULD CHANGE & PRODUCTION STARTUP CHECKLIST                                                                                     .

# GENERAL DETAILS

| Machine ID |  |
| --- | --- |
| Mould Removed |  |
| Mould Installed |  |
| Date |  |
| Start Time |  |
| Setter Name |  |
| Assistant Name(s) |  |

# SECTION 1: MOULD REMOVAL CHECKLIST

| # | Inspection Item | ✔ | ✖ | Comments |
| --- | --- | --- | --- | --- |
| 1 | Machine powered down and locked out (LOTO procedure followed)(QMC-STR-001) |  |  | ________________________________________ ________________________________________ |
| 2 | Mould surface and cavities cleaned (internally and externally) |  |  | ________________________________________ ________________________________________ |
| 3 | All plastic residue, runners, and parts removed |  |  | ________________________________________ ________________________________________ |
| 4 | Water channels blown out with compressed air |  |  | ________________________________________ ________________________________________ |
| 5 | Air and hydraulic lines disconnected safely and labeled |  |  | ________________________________________ ________________________________________ |
| 6 | Wear pads, guide pillars, and moving components cleaned and greased |  |  | ________________________________________ ________________________________________ |
| 7 | Mould condition checked for visible damage |  |  | ________________________________________ ________________________________________ |
| 8 | Nozzles, nipples, and couplings checked for wear/damage |  |  | ________________________________________ ________________________________________ |
| 9 | Area cleared of tools and foreign objects |  |  | ________________________________________ ________________________________________ |
| 10 | Removal documented in system |  |  | ________________________________________ ________________________________________ |

Removed By: ____________________   Signature: ____________________

# SECTION 2: MOULD INSTALLATION CHECKLIST

| # | Inspection Item | ✔ | ✖ | Comments |
| --- | --- | --- | --- | --- |
| 1 | Correct mould verified against production job card |  |  | ________________________________________ ________________________________________ |
| 2 | Mould cleaned and inspected prior to installation |  |  | ________________________________________ ________________________________________ |
| 3 | Mould properly aligned and seated on machine platens |  |  | ________________________________________ ________________________________________ |
| 4 | Centre ring fitted and tightened correctly |  |  | ________________________________________ ________________________________________ |
| 5 | Clamps installed correctly, and tightened |  |  | ________________________________________ ________________________________________ |
| 6 | Water lines connected securely; flow tested |  |  | ________________________________________ ________________________________________ |
| 7 | Hydraulic connections secured and leak-free |  |  | ________________________________________ ________________________________________ |
| 8 | Air lines connected; air valves operational |  |  | ________________________________________ ________________________________________ |
| 9 | Hot runner system connected, zones verified, and set to temperature |  |  | ________________________________________ ________________________________________ |
| 10 | Thermocouples and sensors connected and verified |  |  | ________________________________________ ________________________________________ |
| 11 | Ejector mechanism tested and functioning properly |  |  | ________________________________________ ________________________________________ |
| 12 | Heater bands powered and temperature settings correct |  |  | ________________________________________ ________________________________________ |
| 13 | Machine guards, interlocks, and safety systems tested |  |  | ________________________________________ ________________________________________ |
| 14 | Dry cycle completed without binding or misalignment |  |  | ________________________________________ ________________________________________ |
| 15 | First-off part visually inspected |  |  | ________________________________________ ________________________________________ |
| 16 | All tools accounted for and removed from machine area |  |  | ________________________________________ ________________________________________ |

Installed By: ____________________   Signature: ____________________

# SECTION 3: MOULD RUNNING & PRODUCTION STARTUP CHECKLIST

| # | Inspection Item | ✔ | ✖ | Comments |
| --- | --- | --- | --- | --- |
| 1 | Machine operating smoothly and consistently |  |  | ________________________________________ ________________________________________ |
| 2 | Cycle time matches job card |  |  | ________________________________________ ________________________________________ |
| 3 | Product quality confirmed (shape, dimensions, visual) as per master sample |  |  | ________________________________________ ________________________________________ |
| 4 | Correct labels, stickers, or barcodes applied to product |  |  | ________________________________________ ________________________________________ |
| 5 | No leaks (water/oil) or abnormal noises observed |  |  | ________________________________________ ________________________________________ |
| 6 | Scrap bins and part flow setup properly |  |  | ________________________________________ ________________________________________ |
| 7 | First-off sample signed off and retained by QC |  |  | ________________________________________ ________________________________________ |

# SECTION 4: OVERHEAD INTEGRATED ROBOT STARTUP CHECKLIST ( If applicable )

| # | Inspection Item | ✔ | ✖ | Comments |
| --- | --- | --- | --- | --- |
| 1 | Robot power supply turned on and display operational |  |  | ________________________________________ ________________________________________ |
| 2 | Home position check completed |  |  | ________________________________________ ________________________________________ |
| 3 | All end-of-arm tooling inspected and secured |  |  | ________________________________________ ________________________________________ |
| 4 | Air supply to robot confirmed and regulated |  |  | ________________________________________ ________________________________________ |
| 6 | Robot path clear of obstructions |  |  | ________________________________________ ________________________________________ |
| 7 | Correct program selected and loaded |  |  | ________________________________________ ________________________________________ |
| 8 | Dry run performed |  |  | ________________________________________ ________________________________________ |
| 9 | Integration with IMM (signals and timing) verified |  |  | ________________________________________ ________________________________________ |
| 10 | Full cycle test completed with parts removed successfully |  |  | ________________________________________ ________________________________________ |

# Final Approval Sign-Offs

| Role | Name | Signature |
| --- | --- | --- |
| Supervisor | ____________________________ | ____________________________ |
| Machine Setter | ____________________________ | ____________________________ |
| Quality (QC) | ____________________________ | ____________________________ |

Finish Time: ______________________

Comments / Notes:

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________

__________________________________________________________________________________